Programming indicator for PX-555/666/777/728/628/328/777（Plus）

Welcome:

Welcome to use PUXING programming software! It is software for setting main functions of PUXING 2-way radios. You can easily program your radio by using this indicator.

1. To program PX-777/328 please use “PX-777/328 CH128 MSK+CCIR”.

2. To program PX-777 with CCIR PTT ID version, please use “PX-777/328 CH128 CCIR) ”

3. To program PX-777 with 2 tone version, please select “PX-777/328 CH128 +2 tone”

4. To program PX-728 V1 version, Select “PX-728 V1”

Firstly let us to program the model PX-777.

You need to connect RS232 port of program cable to your computer and connector connect to your PX-777 radio and make sure your radio power ON.

1. install the software with any keys as its serial.

2. Click the desktop icon and select PX-777/328 CH128 MSK+CCIR

3. After you click OK, there will be a frequency table windows which you can program frequency, tone, power, channel name etc.

4. There are four ports com1, com2, com3, com4 option on PORT menu.

Please select one port which suitable for your PC.

5. You can now click READ from radio to read the default radio’s data.

6. After your program the frequency, tone or OPTION FEATURE on PROGRAM MENU, you can click WRITE to write programmed data into radios.

To program data into your radio, fill in the data which you need to program

TYPE: Select your model

TX POWER RANK: from 0-9 (“0” is low power and “9” is high power )

MANUAL SETTING: Enable/Disable input frequency or switch to frequency mode.

RECEIVING (MHZ): RX frequency

TRANSMITTING (MHZ): TX frequency

RX TONE: RX CTCSS or DCS tone (QT/DQT.) (Total 50 CTCSS and 104 DCS invertable)

TX TONE: TX CTCSS or DCS tone (QT/DQT.)

W/N: wide bandwidth or narrow bandwidth channel spacing (Wide=25Khz, Narrow=12.5Khz)

SCAN: YES=Add this channel to scan, NOT=Disable

TX POWER: Low or High output power.

Opt signal: MSK (encode & decode), DTMF (Encode only), CCIR(Encode & decode, set by your dealer)

PTT ID: Enable/Disable PTT ID.

BUSYLOOK: If select on, to enable the busy channel/tone lockout .

Name: to program channel's name

4.       Click program and enter to optional feature.(Test mode and edit date are temporarily unavailable)

VOX Gain level: To enable/disable the Vox function on radio

Priority channel: To select certain channel as priority scan, radio always scans this channel

PTT ID:

NOTES: Please select Msk only, CCIR and 2 tone was added by dealer, your radio may not have these features. Furthermore, CCIR is just use for display PTT ID, it may not work as selective call.

To programming ANI (MSK) you can follow these steps.

- ID is the radio name which can display on screen, you can enter different alphanumeric as radio’s name, you must click “End of TX”

- Call ID  is a code which radio will send, the code must have same numbers as other radios.

- Please don’t fill in any other boxes as they are use for other purpose

- Press OK and Click Write to radio and it display “write radio progress”

- After the PC program, please set the ANI to be ON by keypad (press MENU+0).

When your radio have CCIR as its PTT ID:

- Select “CCIR” of main windows as its OPT signal

- 2. Click PROGRAM, OPTIONAL FEATURES

- Input six numbers as ID and marked ‘End of Tx’, the number MUST be different with other radios

- Input six other numbers as CALL ID

- press OK and write to radios.

- After the PC program, please set the ANI to be ON by keypad (press MENU+0).

Note: Test mode under “program” menu is temporarily unavailable

PX-999

After install px-999 software, there is possibly a Chinese version, you can shift to English.

Click the 5th menu and select English language

PUXING ELECTRONICS CO LTD.

TECHNICAL DEPARTMENT

12 April, 2007